2M 30x HD IP

OUTDOOR PTZ

CAMERA

PTSN-3012-W

: Wall Mount Type

PTSN-3012-C

: Ceiling Mount Type

- Dimension

1. GENERAL FEATURES

- 360x Zoom Control (Optical 30x, Digital 12x)

- 5.7” Bubble dome enclosure

- Endless 360° Pan Rotation

- Lightning Speed (MAX.360°/sec) & Smooth Manual Speed

(MIN. 0.05°/sec)

- Ultra Low Vibration due to S-curved acceleration & deceleration.

- Day & Night (ICR)

- 1024 Preset Positions with Alphanumeric Labels

- 128 Pattern, 128 Scan and 128 Group Functions

- Power-up Action, Parking Action

- Built-in 2 Sensor Inputs and 1 Relay Output

- IP66 environment protection as well as built-in Fan & Heater

- Vandal-Proof

(Dome Cover: PC / Housing & Accessories: AL Die-cast)

- Built-in Fan & Heater

2. PTZ FEATURES

Micro-Step Technology

Due to the Micro-stepping technology, one step angle is divided by 256. Accordingly, high accuracy as well as ultra low sound noise (under 60dB) can be achieved. Micro-stepping technology enables ultra low speed down to 0.05°/s for high zoom operations.

Vector Driving

Based on the shortest time and the smallest cost concept, each Pan and Tilt motion is done with simultaneous and harmonious manner by Vector drive technology.

3. IP FEATURES

Real-time High Definition Speed Dome Camera

Using the state of art video compression engine, real time HD video streams of 1080p and 720p can be delivered for recording and monitoring.

Triple codec, Simultaneous Multi-Streaming

Triple H/W codec enables to transmit Max 3 independent streams with different settings for different requirements. Therefore, it is possible to combine seamless remote video monitoring using H.264 with simultaneous transmission of high quality MPEG-4 to Network Video Recorder. Also, additional MJPEG can be delivered for Mobile devices such as Smart Phone or PDA.

Local SD Card Backup

Some important sequence of still images can be stored in the SDHC memory card. This action can be configured to work with pre-post-  alarm event or a given interval.

4. UNIQUE FEATURES

Virtual Joystick on the Image

Virtual Joystick is inspired by real 3-Axis Joystick. You can decide PT direction by right clicking the image and change the zoom in out by scrolling mouse wheel. The PT speeds are proportional to how much you deviate mouse from the center. Using this function, control of PTZ is done in instinctive manner.

Auto Map Generation

After installing PTZ, make camera to generate the Scenery Map autonomously. It will process and stitch a series of pictures to make one overall Scenery map. Using Scenery map, you can quickly switch one place to another. By drawing a bow on the map, PTZ will move to the center of the box as well as adjust zoom ratio to match the box.

Specification

| CAMERA |  |  |
| --- | --- | --- |
| Image Device | 1/2.8” 2.0 Mega Pixel CMOS |  |
| Effective Pixels | Approx. 2 Mega pixels |  |
| Aspect Ratio | HD: 16:9 SD: 4:3 |  |
| Zoom | X30 Optical Zoom / X12 Digital Zoom |  |
| Focal Length | f = 4.6 ~ 138.0 mm, F1.6 ~ F4.7 |  |
| Horizontal angle | 59.5°(W) ~2.1 °(T) |  |
| MIN. Illumination | Color: 1.7 Lux @ 50IRE, F1.6 B/W: 0.3 lux @ 50IRE, F1.6 |  |
| Day & Night | Auto / Day / Night (ICR) |  |
| Focus | Auto(Near Limit setting)/ Semi Auto/ Manual |  |
| DNR | Level Adjustment(1~5) |  |
| Shutter Speed | 1/1 ~ 1/10000 sec (22Steps) |  |
| AGC | Auto / MANUAL / MAX Gain Limit |  |
| White Balance | AUTO/ATW/Manual/Indoor/Outdoor/One-Push/SodiumVaporLamp(fix/auto) |  |
| BLC | OFF/WDR/BLC |  |
| Image Stabilization | On/Off |  |
| Other Functions | POWER UP ACTION, Parking Action |  |
| PAN / TILT |  |  |
| Range | Pan : 360°(Endless) |  |
|  | Tilt : 90° |  |
| Pan/Tilt Speed | Preset : 360°/ sec |  |
|  | Manual : 0.05°~360°/sec (Zoom Proportional) |  |
|  | Scan : 1°~ 360°/ sec |  |
| Preset | 1024 Presets with Labels |  |
| Pattern | 128 Patterns |  |
| Scan | 128 Scans |  |
| Group | 128 Groups (MAX. 20 Entries for Each Group) |  |
| Privacy Zone | Spherical Privacy Zone Masking |  |
| GENERAL |  |  |  |
| Digital In/Out |  | 2 Alarm Sensor In / 1 Out |  |
| Video Out |  | BNC: VBS 1.0 Vp-p |  |
| Power |  | AC 24V Heater Off : 23W Heater On : 55W High-PoE IEEE 802.3at(40W) |  |
| Approvals |  | FCC, CE, IK10, ROHS, IP66 |  |
| Housing |  | Vandal Proof Structure / Color: Cool Gray |  |
| Material |  | Body: Aluminum Die-casting Clear Bubble: Polycarbonate |  |
| Dimension |  | Net (H): Ø259 x 265.7 (H) mm |  |
| Weight |  | Net: 6 Kg, Package: 10.9 Kg |  |
| Operating Temp. |  | -40°C ~ 50°C / 40°F ~ 122°F |  |

| NETWORK |  |  |
| --- | --- | --- |
| OS |  | Embedded Linux |
| DSP |  | TI DM368 |
| Video Compression |  | Triple Codec,H.264/MPEG4/MJPEG |
| Video Streaming |  | Multi Streaming CBR/VBR (Controllable frame rate and Bandwidth) |
| Resolution |  | H.264 : 1080p/720p/D1/CIF MJPEG : 1080P/720p/D1/CIF/QCIF MPEG-4 : 720p/D1/CIF |
| Frame Rate |  | Max. 30 fps for 1080p/720p/D1/CIF/QCIF |
| Audio Compression |  | Two way, G.711 PCM. μ-law 64kbps 8kHz |
| Local Storage |  | SDHC memory card (Card is not included) |
| Motion detection |  | Notification: FTP, e-mail, Alarm out, JPEG Recording on SD |
| Alarm |  | Pre-Pos Alarm |
| Max No. of Clients |  | 10 |
| Protocols Support | IPv4 | ONVIF, TCP/IP, UDP/IP, RTP(UDP), RTSP, NTP, HTTP,HTTPS, SSL, DNS, DDNS, DHCP, FTP, SMTP, ICMP, SNMPv1/v2c/v3(MIB-2) |
|  | IPv6 | TCP/IP, HTTP, HTTPS, DHCP |
|  | DDNS | Proprietary: NET4C Public DDNS services Suport: www.dyndns.com, www.no-ip.com |
|  | Security | HTTPS(SSL), Digest Authentication(ID/PW) |
| OS Supported |  | Windows 7, Vista, XP |

<Optional Accessories>

Ceiling Mount

(BC04)

Corner Mount (BCO02)

Pole Mount (BP02)

SECUBEST Co., Ltd

145-2, Sangdaewon-dong, Jungwon-gu, Seongnam-si,

Gyeonggi-do, 462-807, Korea.

Tel: +82-31-781 0147 Fax: +82-31-7773445

E-mail: sales@secubest.com